#0. LicheeTang’s virtual machine image uses VirtualBox+Lubuntu1604, the default user is lichee, and the user password is licheepi

#1. There are mainly FPGA source code of E203 and BSP code of e203 in the virtual machine. You can see it in /home/lichee, LicheeTangRV_FPGA is FPGA code, and LicheeTangRV_Code is BSP code. Here we mainly talk about the use of BSP code

#2. Open the terminal (ctrl+alt+t)

cd /home/lichee/LicheeTangRV_Code/sirv-e-sdk

Can be seen:

The BSP directory is mainly defined for paltform and env

software is some sample code

work is the storage location of the tool chain

Then there are some scripts run_**.sh e.g.: run_make.sh demo_gpio

run_clean.sh demo_gpio clear compilation

run_gdb.sh demo_gpio for gdb debugging, cooperate

run_openocd.sh demo_gpio use, need to open the two-terminal interface

run_upload.sh demo_gpio burn code to e203

The general use flow is:

run_make  run_upload

If you need gdb debugging, open the terminal:

- one to execute run_gdb and

- one to execute run_openocd

#3. e203 The FPGA source code seems to have a problem in the virtual machine. It is recommended to compile and download under Windows and so on.